| PRODUCTION SHIFT REPORT Riverbend Foods (illustrative) · Plant 2 · Line 3 · Wed 21 Oct 2026 · Shift A 06:00–14:00 | OEE 73.9% |
| --- | --- |

SHIFT NUMBERS

| TARGET | ACTUAL | % OF TARGET | GOOD UNITS | SCRAP + REWORK | DOWNTIME |
| --- | --- | --- | --- | --- | --- |
| 15,300 | 13,560 | 88.6% | 13,310 | 250 | 54 min |

Availability 88.0% · Performance 85.6% · Quality 98.2%. Biggest loss today was speed, not the jam: the line averaged about 34 bars/min against a rated 40 while running.

STOPS AND ISSUES

| When | Lost | Code | What happened / action |
| --- | --- | --- | --- |
| 09:12–09:46 | 34 min | MECH | Film jam at sealing jaws, station 2. Jaw heater reading 12 °C low; re-set by J. Ferreira. Ticket MT-4471. |
| 11:05–11:17 | 12 min | CHG | Planned changeover 38 g → 40 g wrapper. Ran 4 min over the 8 min standard. |
| 13:20–13:28 | 8 min | MAT | Label reel ran out with no spare staged at the line. Second time this week. |
| All shift | — | SPEED | Line set to 36/min after the jam on QA's advice; never returned to 40/min. |

QUALITY

186 bars scrapped (124 from the jam, 62 seal-check rejects), 64 reworked into new wrap. Seal checks every 30 min: 16 of 16 passed after 10:00. Metal detector test at 06:00, 10:00 and 14:00: pass.

HANDOVER TO SHIFT B

Watch seal temperature on station 2. If it drops below 185 °C, stop and call maintenance (MT-4471 still open).

Return line speed to 40/min once three consecutive seal checks pass. QA (R. Patel) has agreed.

Spare label reel staged at the line at 13:45. Keep one staged at all times.

40 g wrapper loaded, about 3,000 m left, enough for about 2.5 hours.

Shift A supervisor ____________________   Shift B supervisor ____________________   Time ________

Company, people and figures are illustrative.